Item Requirements

○ Article Name: Walkable Floor RF absorbers

○ Type and Quantity

| Item No. | Item | Q'ty |
| --- | --- | --- |
| 1 | 37 cm Height Rectangular-shaped floor RF absorbers | 20 ea |
| 2 | 74 cm Height Rectangular-shaped floor RF absorbers | 10 ea |

○ Latest Delivery Date: 24 weeks from the Effective Date of Contract

○ Requirements

- 37 cm Height Rectangular-shaped floor RF absorbers

  - Rectangular-shaped, 37 cm(height) with 61 cm x 61 cm base

  - Carbon-loaded, urethane-foam absorber

  - Operation temperature: +5°C to +35°C

  - Reflectivity: min. -35 dB @ 9 GHz

  - Fire-retardancy: NRL 8093 Tests 1, 2 and 3 / ISO 11925-2 Class E

  - Quality control by IEEE standard 1128

  - 5 years of RF performance under controlled environment

- 74 cm Height Rectangular-shaped floor RF absorbers

  - Rectangular-shaped, 74 cm(height) with 61 cm x 61 cm base

  - Carbon-loaded, urethane-foam absorber

  - Operation temperature: +5°C to +35°C

  - Reflectivity: min. -35 dB @ 9 GHz

  - Fire-retardancy: NRL 8093 Tests 1, 2 and 3 / ISO 11925-2 Class E

  - Quality control by IEEE standard 1128

  - 5 years of RF performance under controlled environment

- Delivery: Sea-freight available