Request for Proposal
For

Vibro-acoustic modulator
RFP NO : KARI-10-16
December 20, 2010
Korea Aerospace Research Institute
1.
INTRODUCTION

1.1 Introduction

In KARI AITC(Assembly, Integration and Test Center), for the acoustic test, the high intensity acoustic chamber is operated and can generate the noise of 150dB OVSAL. 

This acoustic chamber uses the low frequency source(WAS-3000) and high frequency source (EPT 200) to generate the acoustic noise. The purpose of this project is to change the vibro-acoustic source of high frequency band(EPT 200) as new acoustic modulator of the better performance.

1.2
Scope

This RFP document describes all requirements related to the contract for Vibro-acoustic modulator and also explains the procedures to be undertaken by Bidder seeking to respond this RFP. This RFP consists of following sections.

Section 1
Presents an Introduction

Section 2
Presents Bid Instruction
Appendix A
Terms and Conditions

Appendix B
Statement of Non-compliance Table
Appendix C
Technical Specification

Appendix D
Warranty, Maintenance and Other Requirements

Appendix E      Format of Proposal

2.
Bid Instruction

2.1
Qualification of Bidders
2.1.1
Bidders shall have knowledge and at least five (5) years of experience in designing, manufacturing and delivery of vibration reduction system (Vibro-acoustic modulator). Bidders should submit official document showing of sales and installation records for high frequency acoustic source and acoustic chamber facilities.
2.1.2
The qualifying statement for the requirements and supporting materials shall be provided in the Proposal. Additional qualifying evidence may be requested to Bidders during the evaluation.
2.2
Submittal of Proposal

2.2.1
Proposal Documents

The following information as required for KARI’s evaluation shall be included in the Bidder’s Proposal.
1) Chapter 1: Executive summary of proposal 

2) Chapter 2: Management Proposal
3) Chapter 3: Technical Proposal (refer to Appendix D)
4) Chapter 4: Terms and Conditions Proposal (with Statement of Non-Compliance Table)
5) Chapter 5: Price Proposal (to be submitted in a separate sealed envelope)
             - Price proposal should be a price break down 
6) Chapter 6: Any other documents (Manufacturer and Supplier’s Certificate)
Bidder shall submit Proposal in the same sequence above. If Proposals provided by Bidders are not in the order, the Proposal might be disregarded for effective evaluation.
2.2.2
Number of Copies

One (1) original and five (5) copies of the Proposal shall be submitted. CD-ROM disks (Preferably in PDF format) containing the text of the Proposal shall be additionally submitted from which the price proposal shall be excluded. In case of any discrepancies between the Proposal and CD-ROM version, the original version shall prevail.
2.2.3 Bid Closing Date
The closing time for the Proposal is January 4, 2011, 15:00 Korean Local Time. If any part of the Proposal as required under this RFP is submitted after the stipulated closing time, the entire Proposal will be deemed late. KARI in its absolute discretion may exclude any late Proposal from further consideration.
2.2.4
Method of Proposal Submission
The Proposal shall be submitted to the following address :
Mr. Hang-Seok CHO

Senior administrator of International Contract Department

Korea Aerospace Research Institute
45 Eoeun-Dong, Yuseong-Gu

Daejeon 305-333, Korea

Tel) +82-42-860-2659,Fax)+82-42-860-2666

E-mail) withu4ever@kari.re.kr
2.2.5
Statement of Compliance

Bidder shall submit any “non-compliance” with or “deviations” from the requirements of this RFP including “Terms and Conditions” in the form as presented in Appendix B.
The non-compliance table shall list in numerical order, every paragraph and sub-paragraph of the RFP.

2.2.6
Point of Contact
The communication between KARI and prospective Bidders for contractual or technical matters shall be controlled through the following designee:
Mr. Hang-Seok CHO

Senior administrator of International Contract Department

Tel) +82-42-860-2659, Fax)+82-42-860-2666

E-mail) withu4ever@kari.re.kr
Besides the Proposal submission, technical matters can be communicated with following person:
Mr. Jong-Min LIM
Space Environment Test Department
Tel) +82-42-860-2562, Fax)+82-42-860-2234
E-mail) acoust@kari.re.kr
Bidder shall specify in the Proposal Volume 1 Executive Summary its points of contact for KARI to communicate in connection with the RFP, Proposal and the subsequent evaluation.
Bidder’s information on the points of contact shall include the name, title/position, telephone number. E-mail address, and Fax number.

2.3
Bidding Schedule

The following schedule is a tentative milestone for the eventual Contract and may be changed at KARI’s sole discretion:

       [1]   RFP Release: December 20, 2010
       [2]   Bid Closing Date: January 4, 2011
       [3]   Evaluation of Proposal: January, 2011         

[4]   Contract Negotiation Start: January, 2011
2.4   Validity of Proposal

The Proposal shall be valid for one (1) year from the Bid Closing Date.
2.5   Bid Bond

The Bid Bond is a critical requirement by Korean government regulation. Therefore, the Proposal without Bid Bond will be dropt out in the bidding.
2.5.1
Amount of Bid Bond
Bidder shall establish a bid bond in favor of KARI not later than the Bid Closing Date and submit the bond to KARI along with its Proposal. The total amount of the Bid Bond shall be at least five (5) percent of the total bidding price. The bond shall remain valid until one (1) year after the bid closing date and if the Bid validity is extended, the Bid Bond shall also be extended accordingly.
2.5.2
Bid Bond Establishment Method

The Bid Bond shall be issued by first class international bank in the form of an irrevocable letter of credit and advised through “Woori Bank” (Daejeon Branch at 1495 Dunsan-dong, Seo-gu, Daejeon, Korea, Tel:+82-42-488-9704, Ext.320/ Fax:+82-42-488-8913). The bond shall be available by KARI’s draft at sight drawn on the bank, accompanied by the statement of KARI stating that Bidder has refused to enter into a contract with the terms and conditions in the proposal as amended, if any, that Bidder has hindered the fair bidding by fraud or mispresentation, or that Bidder has withdrawn its Proposal after the Bid Closing Date.
2.5.3
Bid Bond Return
All of the Bid Bonds will be released promptly at the time of contract award by KARI without interest.
2.6
Questions and Modifications

2.6.1
Any questions as to the intent and clarity of the RFP may be asked to KARI’s point of contact as specified in Article 2.2.6 by email, fax or letter no later than ten (10) days
before the Bid Closing Date. After the Bid Closing Date, Bidder shall not amend, Modify, or supplement the Proposal, or submit any other documents to KARI, except with KARI’s written request. 
2.6.2
KARI may ask each Bidder in writing for clarifications, amendments, revisions, or supplementation of the Proposal and any other documents submitted by Bidder at any time before the execution of the Contract. KARI may also ask each Bidder to submit and supply additional information or documents for evaluation of the Proposal.

2.7
Non Contractual Obligation

2.7.1
This RFP shall not be interpreted as an offer for a contract or any type of guarantee by KARI. It is KARI’s sole discretion whether or not to allow the award to any Bidder. KARI owes no responsibility or obligation to select the Contractor among Bidders.
2.7.2
This solicitation for the Proposal shall neither be construed to create an obligation on KARI to enter into a contractual instrument, nor shall it serve as a basis for any claim for reimbursement of costs for the effort to prepare, submit, amend, revise or supplement the Proposal, and other related documents expended by Bidder regardless of whether or not Bidder is successful in receiving an award as a result of the solicitation.

2.8
Acceptance/Rejection of Bids

KARI reserves the right to accept or reject any Bidding, and to amend the Bidding process and reject all Bidding at any time prior to the award of the Contract without thereby incurring any liability to the affected Bidder(s), nor is KARI under any obligation to inform the affected Bidder(s) of the ground for its action.
2.9
Selection of Successful Bidder
2.9.1
Proposal submitted by Bidders will be evaluated based on evaluation process. 
2.9.2
The Successful Bidder will be selected by the evaluation committee among the Bidders after the evaluation of the Proposals. any Bidders shall not raise any objection regarding the proposal evaluation committee’s decision thereupon.
2.9.3
If KARI and the Successful Bidder fail to agree on the terms and conditions for the eventual Contract, KARI may terminate the contract negotiation with the bidder without any responsibility or obligation and proceed to have a re-biding.

2.10
Contract

The Contract will be effective subject to the KARI’s approval thereof. The Draft Terms and Conditions as attached in Appendix A. is the baseline requirement for the Contract negotiation with the Successful Bidder, and will be the basis of the Contract.
2.11
Language 

The Proposal and any documents submitted to KARI under the RFP and communication and the Contract language shall be in English.
2.12
Certificate and Government Approvals

Bidder shall receive the government approval and certificates (e.g. export licenses) in relevant countries, necessary for Bidder’s work for the RFP.
2.13
Confidentiality

There shall be no news release, public announcements, denials or confirmation in connection with this RFP or Contract award without the prior written approval of KARI. All documentation submitted in response to this RFP shall be marked “COMMERCIALS-IN-CONFIDENCE” on the first page of each document. All elements of the RFP shall be kept confidential, and shall not be intentionally disclosed by Bidder to any Third Party.
2.14
Withdrawal of Proposal

Bidder may withdraw its Proposal at any time before the Bid Closing Date. Withdrawal shall be made in writing, and shall be received by KARI before the Bid Closing Date.
2.15
Property of Proposal

All documents submitted in response to the RFP shall become the property of KARI and will be retained by KARI.
2.16
Expenses

Expenses incurred by Bidder in connection with the preparation, submittal, and any subsequent clarification or negotiation activities are for its own account and will not be reimbursed by KARI.
2.17
Governing Law

This RFP and the Contract shall be construed in accordance with and governed by the laws of the Republic of Korea.

2.18
Arbitration

Any disputes in relation to, and under the RFP and the Contract shall be resolved through arbitration in Seoul, Korea under the Rules of the Korean Commercial Arbitration Board.

Appendix A.

Terms and Conditions

Refer to Attached Word File
Appendix B. 

Statement of Non-compliance Table

	KARI’s General Terms & Conditions
	Bidder’s sales standard
	Deviation

	Article No.
	Description
	
	

	
	
	
	

	
	
	
	

	
	
	
	

	
	
	
	


Appendix C. 

Technical Specification
1. Quantity 
High frequency acoustic source(modulator) : 2 Sets
2. Specifications
For the acoustic facility, main components are :

      - Reverberant chamber,

      - GN2 supply & control system 

- Acoustic horns & modulators

      - Acoustic control system

      - Monitoring & Safety system

Figure 1 shows a block diagram of a acoustic facility

[image: image1.png]Air Handing
Unit
Acoustic

Modulator
/Horns

Reverberation
Chamber

Specimen
Door

GN2 Supply
System

Isolation
System




[image: image2.jpg]Mufiler

And Fan Pressure Regulation
Enclosure

Main Control
Consale Building
Monitoring 113 Oct. Filter
Systom Random
. Signal
Safety ; 113 Oct. Filter
InterLock
Data Acquisition System
{ Backup Data Acquisition
Cooling |
Tower | [ PCBased
GN2 - 1| _ Signal
Vaporizer i | Generation
LNz ;
Boiler AHU |





Figure 1 Block diagram of a acoustic facility

KARI acoustic chamber employ the two horns which have cutoff frequencies of 25Hz and 125Hz are employed for acoustic impedance matching. Two type of modulators are used to generate the sound. One is the WAS-3000 which is used at 25Hz horns. The other is EPT 200 which is used at 125 Hz. Figure 2 shows the acoustic noise sources of KARI.

[image: image3.emf]
Figure 2 Acoustic noise source in KARI acoustic chamber

Figure 3 shows the vibro-acoustic source of high frequency band(EPT 200) which is installed at KARI.

[image: image4.emf]
Figure 3 EPT-200(High frequency vibro-acoustic modulator)

The purpose of this project is to change the vibro-acoustic source of high frequency band(EPT 200) as new acoustic modulator of the better performance.

2.1
Acoustic electro-pneumatic noise source

2.2
Acoustic power output : 10 kW

2.3
Air flow rate for 10kW : 1,500 scfm

2.4
Air pressure required at exhaust plenum housing : 30 psi

2.5
Frequency response : 20 to 5000Hz (Maximum modulation up to 1250Hz)

2.6
Drive circuit maximum current : 60 amp rms(sine), 40 amps rms (random)

2.7
Generate spectrum defined as an overall-SPL from 113dB to over 160dB at KARI acoustic chamber(Frequency range : 315Hz ~ 10kHz)

2.8
To show the capability of acoustic modulator, the supplied acoustic source shall have the more acoustic power than EPT 200 and submit the measurement results such as below(Frequency range : 315Hz ~ 2kHz)

Table 1. Example for acoustic noise capability
[image: image5.emf]
[image: image6.emf]
Figure 4 Acosutic noise generation capability of EPT 200

Appendix D.

Warranty, Maintenance and Other Requirements
1. Warranty

1.1 The system shall be warranted for duration of 2 years from the date of final acceptance on the site against any operational faults. The date of acceptance is the date when the whole system is completely installed and its function is approved perfect in a written format by KARI or its representative.

1.2 The warranty covers the replacement of faulty parts, the labor required for repair, traveling cost, and all necessary costs. 

1.3  The contractor or A/S teams in a foreign country shall submit the written plan for repair within 4 days while the contractor or A/S teams in Korea shall do it within 3 days after the receipt of the notification of fault. The contractor or A/S teams in a foreign country shall completely finish the repair within 2 weeks after receipt of the notification of fault while the contractor or A/S teams in Korea shall do it within 1 week. 

1.4  To meet these requirements, the contractor shall propose a detailed plan for the prompt repair in the proposal. The detailed information (location, number of crews, resume, phone and FAX numbers, etc.) of the A/S teams (in Korea and foreign countries) shall be shown in the proposal and shall be approved by KARI.

2. Maintenance

2.1 General Rules

All parts requiring reconditioning or maintenance shall be easily accessible.  
Their installation and removal shall be easy : i.e.

- the installation and removal shall be possible with standard tooling or provided specific handling tooling,

- visual checks shall not be obstructed (lighting shall be provided if required),

- Removal shall be performed in perfect safety (electric shocks, high voltage, water pressure, compressed air, etc.)

All the parts shall be visibly identified. Unless there is a particular reason, the part used in the subassemblies shall b standard and available at the various manufacturers for a duration of 10 years.

A maintenance note shall be included in the documentation by using the friendly explanation with reference to schematic diagrams, drawings or photographs :

- A maintenance schedule and guide providing detailed instructions of disassembly, adjustment and calibration of the units, with a list of the tools necessary for carrying out these operations.

- A trouble shooting guide listing the main failures for the various systems and units.

2.2 Spare parts

The contractor shall be responsible for providing the spare parts required to maintain the whole system for a period of 10 years. The contractor shall update the list of spare parts in the event of modifications. 

The contractor shall draw up a list of mandatory spare parts (due to long lead time, redundancy, safety, etc.) and a list of optional spare parts with their failure frequency records.

The contractor shall be responsible for answering immediately the operational and maintenance questions which KARI brings in by FAX or telephone calls until the system is dismantled by KARI. General description

Appendix E. 

Format of Proposal

The proposal shall consist of 10 Work Packages (WP).


WP 1   : Bidder's Qualification Document


WP 2   : Overview and Technical Proposal


WP 3   : Warranty Proposal


WP 4   : Maintenance Proposal


WP 5   : Transport - Handling - Storage Proposal


WP 6   : Acceptance Test Plan Proposal

WP 7   : Documentation

The bidder shall present 6 copies of the proposal. A4 or letter size paper shall be used. The bidder is requested to strictly follow the following illustrated format (tabular form). This is very important to compare and evaluate different bidders’ proposal. Consequently, disregard for this rule may end up with disqualification. Also, well-prepared and well-organized proposal will be highly appreciated and evaluated. The landscape orientation will be acceptable for tables. The suggested contents in each WP are minimum requirements and works for reminiscences. Ambiguous expression will act adversely to the bidder. 

Note : The prices which are used in the proposal to calculate the final total and breakdown prices shall be distinguished from the other prices (e.g. optionally suggested parts by the bidder or KARI, . . . ) by marking a.p. (applied price) after the prices. In other words, the sum of a.p. shall be the final total price proposed in the proposal.

(example)

	Item
	Price
	Unit
	Subtotal

	Item #1
	$ 5,000 a.p.
	2
	$ 10,000

	Item #2
	$ 4,000 a.p.
	1
	$ 4,000

	Item #3
	$ 2,000 a.p.
	3
	$ 6,000

	:
	:
	:
	:

	
	
	Total
	$ 20,000


WP 1 : Bidder's Qualification Document

The followings shall be included in this WP.

(a) Bidder's last 5 years experience on the prescribed system in this "Request for Proposal" including :

(1) design

(2) manufacturing 

(3) installation (specifications, performance, purchaser, supplied year, and price)

(4) training (operation and maintenance) of the facility for foreign countries

(b) Supporting program to maintain high quality of the equipment

(c) Bidder’s Qualification Evidence Documents to show qualification on bidder in this RFP. It may be generated by the ON-SITE Users(can be requested by KARI). The documents include personnel's information in charge of the facility, address, FAX number and the others from the world well known satellite testing institutes or companies which are using the facilities installed by the bidder

(d) Other information


1. Total Employees

      

- Total number of Employees (Engineer / Technician)

      

- Number of Employees for the directly related area

  
2. Plant Scale

      

- Total area of space

      

- Factory & Building area

   
3. General brochure

4. Any documents with which the bidder can show his/her financial and technical abilities (e.g. technical awards, bank balance, etc.)

WP 2 : Overview and Technical Proposal

Concise and clear expression is required. A long sentence should be broken down into several sentences. Detailed block diagrams, figures and photos are recommended for clear illustration.

The bidder should prepare tables for the check list to show the bidder's compliance with the "Request for Proposal". The bidder’s compliance list shall be legal bind. This table shall be located in the front of Overview part. The article numbers in the "Part I. Overview" and "Part II. Technical Requirements" in the RFP should be completely enumerated in the table. The table format to be followed is as follows:

Check List for "Part I. OVERVIEW", "Part II. TECHNICAL REQUIREMENTS", “Part Ⅲ. Warranty, Maintenance and other Requirement” and “Part Ⅳ Terms and Conditions”(COMPLIANCE LIST)

ANR
: Article Number in the Part I and Part II in the RFP

Y/N
: Compliance (Yes or No)

CAP
: Corresponding Article number in the bidder's Proposal to ANR

AAM
: Article number of Added Material in the bidder's proposal which is not mentioned in this RFP

(Exemple)

	ANR
	Y/N
	CAP
	Remark
	AAM
	Remark

	I-1.
	yes
	I-1.1
	Ditto
	
	

	I-2.
	yes
	I-2.1.1
	Ditto
	
	

	I-2.1.
	yes
	I-2.1.2.
	Ditto
	
	

	…...
	……
	……
	
	
	

	II-1.
	yes
	II-1.1
	Ditto
	
	

	II-2.
	yes
	II-2.1.2.
	Ditto
	
	

	II-2.1.
	yes
	II-2.1.3.
	Ditto
	
	

	II-2.1.1.
	yes
	II-2.1.5.
	The specifications look old. We updated them
	
	

	N/A
	N/A
	N/A
	N/A
	II-2.1.4.
	We currently developed a new method.

	II-2.1.2.
	yes
	II-2.2.1.
	Ditto
	
	

	II-2.1.3.
	no
	II-2.2.2.
	The resolution is high above the necessity.

The current technology for the system .............
.……...
	
	

	II-2.1.4.
	yes
	II-2.3.1.
	Ditto
	
	

	II-2.1.5.
	yes
	II-2.3.2.
	Ditto
	
	

	II-2.2.
	no
	II-2.3.3.
	This specification is not 

Compatible with II-2.2.2 in the Proposal. We suggest other Approach.
	
	

	II-2.2.1.
	yes
	II-2.3.4.
	Ditto
	
	

	II-2.2.2.
	no
	II-2.3.5.
	Ditto
	
	


WP 3 : Warranty Proposal

3.1. Baseline warranty (2 years)


- plan for the prompt repair

- information on A/S team 

WP 4 : Maintenance Proposal

4.1. General description

4.2. Spare parts

WP 5 : Transport-Handling-Storage Proposal (baseline price)


- plan for packaging


- list of special parts and their treatment plan


- storage plan of products for final integration

- price tables

Note : The abbreviation a.p. shall be marked after the appropriate prices in the tables.

WP 6 : Acceptance Test Plan Proposal

6.1. General description

6.2. Acceptance test


- schedule

- test list and procedure

8.3. Back-up plan

For the case that the bidder can not meet the requirements or other contractual binds after the contract is awarded, the bidder shall proposed the back-up plan,
detailed plan and schedule

WP 7 : Documentation

7.1. Documentation


- document list (contents, number of page and figure) to be delivered

2

