Item Requirements

○ Article Name: RF Power Amplifier with external directional coupler

○ Type and Quantity

| Item No. | Item | Q'ty |
| --- | --- | --- |
| 1 | RF Power Amplifier - Frequency range : 4 kHz – 400 MHz - Output Power (Pn min / typ): 60 W / 75 W (4 kHz – 9 kHz) 150 W / 200 W (9 kHz – 400 MHz) - Power output at 1dB compression : 150W min. (9 kHz – 400 MHz) Includes: - External Dual Directional Coupler ∙ Frequency range : 9 kHz – 400 MHz ∙ Power (Pmin, W) : 200 W ∙ Coupling (dB) : 40 ± 0.5 dB | 1 set |

○ Latest Delivery Date: 120 days from the Effective Date of Contract

○ Requirements

- RF Power Amplifier

  - Standard Specifications

    - Frequency range : 4 kHz – 400 MHz

    - Output Power (Pn min / typ): 60 W / 75 W (4 kHz – 9 kHz)

150 W / 200 W (9 kHz – 400 MHz)

    - Power output at 1dB compression : 150 W min. (9 kHz – 400 MHz)

    - Gain (min / typ) : 47.8 / 51 ± 3 dB (4 kHz – 9 kHz)

51.8 / 54 ± 2 dB (9 kHz – 400 MHz)

    - Harmonics (2nd / 3rd) : 20/20 dBC

    - Line Power : 1250 VA

    - Input Power : 0 dBm (1 mW) max.

    - Overdrive Protection : up to +10 dBm for no damage

    - Input Impedance : 50 Ω nominal

    - Output Impedance: 50 Ω nominal

    - Input VSWR : <2:1 typ.

    - Load VSWR : PN –0.5 dB min. at VSWR 2:1; infinite for no damage

    - Spurious (at PN) : -50 dBC typ. (excluding harmonics)

  - General Specifications

    - RF Input : N-f, standard on rear panel

    - RF Output : N-f, standard on rear panel

    - Mains Supply : 200 – 240 V AC ± 10% / 47-63Hz

    - Elapsed Time Meter : via status display

    - Ambient Temperature : 0 ... +45 °C

    - Storage Temperature : -20 ... +85 °C

    - Relative Humidity : up to 95 % (non-condensing)

    - Operating Altitude : up to 2000 m above sea level

    - Vibration and Shock : MIL_STD-810G

    - Cooling : Forced air with integral blower air intake from front, air exhaust              at rear

    - Dimensions (H, D; 19" system) : 3HU, 630mm

    - Weight : 27 kg

- External Dual Directional Coupler

  - Standard Specifications

    - Frequency range : 9 kHz – 400 MHz

    - Power (Pmin, W) : 200 W

    - Coupling (dB) : 40 ± 0.5 dB

    - Insertion Loss (max, dB) : 0.5 dB

    - Directivity (min, dB) : 20 dB

    - VSWR (max.) : 1.25 : 1

  - General Specifications

    - Main Line Connector : N-f

    - Coupling Line Connector : SMA-f

- All Standard accessories should be supplied

- One year warranty