Item Requirements

○ Article Name: 4-Port Network Spectrum Analyzer with Software

○ Type and Quantity

| Item No. | Item | Q'ty |
| --- | --- | --- |
| 1 | 4-Port Network Spectrum Analyzer - Spectator Software - Detector Software | 4 |

○ Latest Delivery Date: 28 weeks from the Effective Date of Contract

○ Requirements

- 4-Port Network Spectrum Analyzer

  - Main function: The device must include a function to monitor the signal at a remote location and to provide accessibility to the monitoring data through a network connection without additional server.

  - Standard Specifications

    - 4/8 user-selectable input ports. Standard 19” 1RU

    - Full satellite L-band, standard C-band, plus cable & wireless bands from 5 MHz to 6.5 GHz (50 Ohm SMA)

    - Built-in Cross-Pol Isolation

    - External 10 MHz reference or internal reference

    - Web browser or API control

    - SNMP status interface

    - USB port allows attached data storage

  - Physical Interfaces

    - RF Input: 4 x SMA, 50 ohms

    - Control: RJ-45

    - Reference: BNC, 50 ohms

    - AC Power: IEC 60320

  - RF Input

    - Input Frequency Range: 5 MHz to 6.5 GHz (with 50 Ohm SMA)

    - Useable Dynamic Range: -110 to +5 dBm (aggregate)

    - Noise Floor

      - -130 dBm/Hz (typical at max atten)

      - -150 dBm/Hz (typical at min atten)

    - Phase Noise (worst case at 6 GHz)

      - - 80 dBc/Hz at 1 kHz offset

      - - 95 dBc/Hz at 100 kHz offset

      - -125 dBc/Hz at 1 MHz offset

    - Maximum Safe Input: +10 dBm

    - Input Isolation (port to port): 55 dB (min)

    - Input Return Loss: -15 dB (min)

  - Measurements

    - Amplitude Accuracy:

      - +/- 0.5 dB (at 25°C)

      - +/- 1.0 dB (0 to 40°C)

    - Frequency Accuracy: +/-2.6 ppm (internal) or as per external

    - Frequency Resolution: 1 Hz

    - Resolution Bandwidth: 1 Hz to 15 MHz

    - Analysis Bandwidth: up to 260 MHz

  - Spurious

    - Images: < -55 dBc (typical)

    - Aliasing: < -55 dBc (typical)

    - DC Offset: < -30 dBc (typical)

    - Averaging: up to 255 averages

  - Measurement Speed

    - 500 MHz span, 1 MHz RBW: 200 ms

    - 200 MHz span, 30 kHz RBW: 630 ms

    - 80 MHz span, 100 kHz RBW: 170 ms

    - 3.5 MHz span, 8 kHz RBW: 90 ms

  - Other Specifications

    - Reference Input: 10 MHz, -5 dBm to +13 dBm, +3 dBm to +13 dBm (auto-sensing)

    - Control Interface: TCP/IP, API, SNMP, HTTPS

    - Power Requirements: 90-264 VAC, 50/60 Hz, 25W

    - Operational Temperature: 0 to 40°C

  - MODCOSs Supported:

    - DVB-S: QPSK ½, 2/3, ¾, 5/6, 7/8 DVB-S2: QPSK ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 5/6, 8/9, 9/10; 8PSK 3/5, 2/3, ¾, 5/6, 8/9, 9/10; 16APSK 2/3, ¾, 4/5, 5/6, 8/9, 9/10; 32APSK ¾, 4/5, 5/6, 8/9, 9/10

    - DVB-S2: QPSK ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 5/6, 8/9, 9/10; 8PSK 3/5, 2/3, ¾, 5/6, 8/9, 9/10; 16APSK 2/3, ¾, 4/5, 5/6, 8/9, 9/10; 32APSK ¾, 4/5, 5/6, 8/9, 9/10

    - DVB-S2X: QPSK 13/45, 9/20, 11/20; 8APSK 5/9L, 26/45L, 23/36, 25/36, 13/18; 16APSK 1/2L, 8/15L, 5/9, 26/45, 3/5, 3/5L, 28/45, 23/36, 2/3L, 25/36, 13/18, 7/9, 77/90; 32APSK 2/3L, 32/45, 11/15, 7/9

- Test report to prove that all performances are satisfied should be supplied.

- All Standard accessories with software should be supplied

- Two year warranty