“Specification of K-type Feedthroughs for a Thermal Vacuum Chamber”

Plug/Body/Thread/Receptacle (PBTR) Type

1 3/4” - 18

Thermocouple Qty. in one feedthrough: 20 channels

Insulation: Teflon

Connector type: Miniature Circular

Coupling: Bayonet

MIL Spec: MIL-C-83723 Series 1, MIL-C-26482 Series 2

Include mating connectors, contacts, and cable clamps

Electrical Hipot at 150 VAC

MSLD Leak Limit: -80 cc He/sec.

Air Side: Not-threaded

1) Conductor: Plug with pins (PLUG/PINS)

2) MS Plug: MS3476L20/41P

Vacuum Side: Threaded

1) Conductor: Receptacle with sockets (REC/SKTS)

2) MS Receptacle: MS3471L20/41S

Operating Temp.: -40 degree F ~ 225 degree F

Material: Stn Stl

O-ring: Nitrile