Item Requirements

○ Article Name: Purchase of an Electrical Simulator for ground test of 6-DOF propulsion system for 6U nanosatellite

○ Type and Quantity

| Item No. | Item | Q'ty |
| --- | --- | --- |
| 1 | Electrical Simulator of 6-DOF propulsion system for 6U nanosatellite Includes : - Flight representative interface and control electronics board - Components to simulate 6 thrusters and relevant parts of the storage and feed system. - Software package to control the 6-DOF propulsion system. - Support | 1 set |

○ Latest Delivery Date: February 21st, 2020

The contractor for the Electrical Simulator shall be capable of providing Flight Models of the 6-DOF propulsion system under a separate contract to be placed next year.

○ Requirements

The following requirements shall be applicable for the 6-DOF propulsion system and the Electrical Simulator shall be aligned with these requirements where applicable:

- The propulsion system shall be capable of providing 6-DOF motion of a 6U nanosatellite (this means translation along and rotation around all 3axis)

- Size: Less than 2U (20x10x10cm) of total size

- Mass: Less than 2kg

- Mechanical Interface: The propulsion system must be suitable for standard 6U cubesat shape (Redesign of the structure should be avoided)

- Electrical interface:

  - Communication shall be possible over these data buses: CAN, I2C, UART

- Performance:

- Max thrust: 20mN for each axis

- Total Delta-V: around 16m/s (assuming a 10kg nanosatellite)

- Capability to measure delivered impulse by each thruster in near real time

- Quality & reliability:

  - The propulsion system shall have flight heritage or equivalent qualification tests performed

  - Capability of meeting radiation requirements equivalent to 5 years in LEO (20 krad)

  - Propulsion system design shall be compliant to the tailored ECSS standards for IOD cubesat projects

  - All electrical components shall be de-rated according to ECSS standards

  - The propulsion system shall have redundant sensors for all critical components

The Electrical Simulator shall mimic the FM module in a representative way, including communication, thruster performance, thruster diagnostics, timing and sequencing of commands, monitoring of propellant tanks and feed system. More specifically, the Electrical Simulator shall have a fully flight representative interface and control electronics board, including identical connectors, so that the Electrical Simulator can replace the FM module in a satellite simulator or a flat-sat system.

○ Payment terms

Payment for the contract shall be made as follows:

| Ratio | Payment terms |
| --- | --- |
| 50% ± 5% | After signing the contract |
| 50% ± 5% | Deliver of all parts |