Item Requirements

○ Article Name: S-band Frequency Up-converter, S-band Frequency Down-converter

○ Type and Quantity

| Item No. | Item | Q'ty |
| --- | --- | --- |
| 1 | S-band Frequency Up-converter | 1 |
| 2 | S-band Frequency Down-converter | 2 |

○ Requirements

S-band Upconverter

- IF Characteristics

● IF Center frequency: 70 MHz

● Impedance: 50 Ohm

● VSWR: <1.3

- RF Characteristics

● Frequency: 70MHz --> 2025 MHz to 2120 MHz

● Frequency setting steps: 1 Hz

● Frequency reference: 10 MHz, internal or external

● Output power: ±10dBm @ max gain

● 3rd order intercept point (IP3): ±20dBm @ max gain

● AM/PM conversion: 0.25°/dB @ Pout= -10dBm

● Phase Noise (@frequency offset from carrier, typical)

: > 65 dBc/Hz @ 10 Hz

: > 75 dBc/Hz @ 100 Hz

: > 80 dBc/Hz @ 1 kHz

: > 90 dBc/Hz @ 10 kHz

: > 100 dBc/Hz @ 100 kHz

: > 120 dBc/Hz @ 1 MHz

● Impedance: 50 Ohm

● VSWR: <1.3

- Interfaces

● RF: N female

● IF: BNC female

● Ext. 10 MHz ref. input: SMA female

● RS485 M&C interface: SUBD, 9 pins male

● Ethernet M&C interface: RJ45

● Monitor output: SMA female

● LO monitor: SMA female

- Transfer Characteristics

● Conversion sense: Positive – No inversion

● Image rejection: >80 dB

● LO Leakage: < -70dBm

● Mute (RF on/off): > 60 dB

● Maximum gain: ≥35 dB

● Noise figure @ max gain: ≥18 dB

- Mechanical Characteristics

● Dimension: 1U

● Power consumption: <150 VA

● Power supply: Available 220 VAC, 60Hz

● Temperature operating: 0°C to 40°C

● Temperature storage: -30°C to +60°C

● Relative humidity operating: 0 to 75%

● Relative humidity storage: 0 to 85%

S-band Downconverter

- IF Characteristics

● IF Center frequency: 70 MHz

● Impedance: 50 Ohm

● VSWR: <1.3

- RF Characteristics

● Frequency: 2200 MHz to 2300 MHz --> 70 MHz

● Frequency setting steps: 1 Hz

● Frequency reference: 10 MHz, internal or external

● Output power: ±10dBm @ max gain

● 3rd order intercept point(IP3): ±20dBm @ max gain

● Phase Noise(@frequency offset from carrier, typical)

: >65 dBc/Hz @ 10 Hz

: >75 dBc/Hz @ 100 Hz

: >80 dBc/Hz @ 1 kHz

: >90 dBc/Hz @ 10 kHz

: >100 dBc/Hz @ 100 kHz

: >120 dBc/Hz @ 1 MHz

● Impedance: 50 Ohm

● VSWR: <1.3

- Interfaces

● RF: N female

● IF: BNC female

● Ext. 10 MHz ref. input: SMA female

● RS485 M&C interface: SUBD, 9 pins male

● Ethernet M&C interface: RJ45

● Monitor output: SMA female

● LO monitor: SMA female

- Transfer Characteristics

● Conversion sense: Positive – No inversion

● Image rejection: >80 dB

● LO Leakage: <-70dBm

● Maximum gain: ≥40 dB

● Phase tracking <10 deg. RMS between any channels

● Noise figure @ max gain: ≥12 dB

- Mechanical Characteristics

● Dimension: 1U

● Power consumption: <150 VA

● Power supply: Available 220 VAC, 60Hz

● Temperature operating: 0°C to 40°C

● Temperature storage: -30°C to +60°C

● Relative humidity operating: 0 to 75%

● Relative humidity storage: 0 to 85%