사양설명서(Technical requirement)

| 품 명 | 국문 : 한국형발사체 1단 추진제탱크 Y-frame 제작용 소재((Procurement of ring forged for propellant tank Y-frame making)) |
| --- | --- |
| 모 델 명 | 해당사항 없음 |
| 세부 사양 | ○ Title : AMS 4144 AL2219-T851 Ring Forged (Rough Machined) ○ Size : 3,587(O.D.) X 3,247(I.D.) X 356 (H) millimiters ○ Roundness : max. 2.5mm ○ Q'ty : Total 10 EA ○ Delivery Item. : - Ring 10 EA - Include Ultrasonic inspection sheet (Class A) - Including a Dummy Material for the tensile stress test - Commercial Packing - Measuring inner and outer diameter before and after T851 heat treatment - Submiting Inspection sheet ○ Required Delivery Date : - 2 EA or more within 13 weeks Ex-Work after receiving P.O, - 8 EA or less within 20 weeks Ex-Work after receiving P.O |

작성일자 : 2017. 6. 22